FIGURE 1 -       Chemical structures of the principal N-PAH existing in the atmosphere.

FIGURE 2 -       Isomer-specific generation and/or emission of nitrated fluoranthenes and

pyrenes in the atmosphere

# TABLE 1 -   Concentrations of NPAH measured in atmospheric aerosols collected in various

locations of Italy, Europe, USA, Brazil (BRA), Algeria (ALG) and Syria (SY).  Data are reported in pg m-3.

| urban area | compound | 2-NFA | 1-NPY | 2-NPY | R1 | R2 | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | location & time | pg m-3 | pg m-3 | pg m-3 |  |  |  |
| Augusta (Syracuse), I | in, wi-sp ‘91-‘92 | 11810 | 5900 | n.ev. | 2.0 | n.ev. | 35 |
| Augusta (Syracuse), I | in, su-au ‘91-‘92 | 6760 | 2240 | n.ev. | 3.1 | n.ev. | 35 |
| Genoa, I | in, su ‘98 | 215 | 70 | 25 | 3.1 | 10.1 | 50 |
| Milan, I | st, wi ‘90-‘93 | 1435 | 405 | 535 | 6.5 | 5.2 | 48 |
| Milan, I | st, su ‘90-‘91 | 230 | 30 | 40 | 8.0 | 6.0 | 48 |
| Naples, I | st, wi ‘93 | 270 | 50 | 70 | 5.4 | 3.9 | 19 |
| Turin, I | st, ’96 | 297 | 163 | n.ev. | 2.1 | n.ev. | 36 |
| Rome, I | re, wi ‘87-‘98 | 190 | 74 | 52 | 2.7 | 5.5 | 48,47 |
| Rome, I | re, su ‘87-‘98 | 244 | 34 | 47 | 4.4 | 7.6 | 48,47 |
| Castelporziano (Rome) | ru, sp ‘92-‘94 | 60 | 4 | 21 | 18 | 4.0 | 19 |
| Castel Romano (Rome) | in, wi ‘00 | 140 | 18 | 46 | 7.8 | 3.0 | tw |
| Lunghezza (Rome) | ru/in, wi ‘00 | 46 | 19 | 29 | 2.4 | 1.6 | tw |
| Malagrotta (Rome) | ru/in, wi ‘00 | 56 | 8 | 32 | 6.8 | 1.8 | tw |
| Montelibretti (Rome) | ru, ‘92-‘00 | 21 | 3 | 3 | 9 | 6.5 | 48,tw |
| Athens, GR | re, ‘96 | 87 | 62 | 40 | 1.9 | 2.1 | 52 |
| Birmingham, UK | re, wi ‘95-‘96 | 221 | 90 | n.ev. | 2.5 | n.ev. | 53 |
| Barcelona, SP | re, ‘94 | 98 | 25 | 20 | 3.5 | 7.7 | 18 |
| Madrid, SP | st, wi ‘99 | 172 | 71 | 40 | 2.3 | 3.7 | ud |
| Madrid, SP | re, wi ‘99 | 69 | 40 | 17 | 4.1 | 4.5 | ud |
| Madrid, SP | re, su ‘90 | 70 | 20 | 40 | 4.8 | 4.1 | 19 |
| Copenhagen, DK | st, ‘98-‘99 | 91 | 127 | 20 | 0.6 | 4.0 | 53 |
| Risø, DK | ru, ‘98-‘99 | 60 | 30 | 8 | 2.3 | 8.0 | 53 |
| Torrance, CA,USA | re, wi ‘86 | 440 | 45 | 45 | 10 | 10 | 54 |
| Claremont, CA, USA | re, su ‘85 | 178 | 20 | 3 | 9 | 60 | 54 |
| Columbus, OH, USA | re, ‘91 | 60 | 20 | n.ev. | 3.0 | n.ev. | 54 |
| Alta Floresta, BRA | fo, wi ‘93 | 43 | 6 | 22 | 2.0 | 7.5 | 19 |
| Sao Paulo, BRA | re, wi’93 | 139 | 34 | 46 | 7.8 | 3.9 | 19 |
| Damascus, SY | Re, wi ‘94&’95 | 243 | 157 | n.ev. | 1.6 | n.ev. | 54 |
| Algiers, ALG | re, ‘99 | 260 | n.ev. | 24 | n.ev. | 11 | 55 |
| Bab Ezzouar, ALG | in, su ‘99 | 1490 | 730 | 220 | 3.1 | 7.7 | 55 |
| Ouargla, ALG | in, au ‘99 | 1240 | 400 | 600 | 3.2 | 2.1 | 55 |

Symbols: 	2-NFA: 2-nitrofluoranthene;  1-NPY: 1-nitropyrene,  2-NPY: 2-nitropyrene.

R1 = [2-NFA]/[1-NPY]; R2 = [2-NFA]/[2-NPY];